| 06-7174 | Valve spring set, 500/600 Twin | =4x NM19302 + 4x NM19303 |
| --- | --- | --- |
| 06-7070 | Valve spring set, 650/750/850 Twin | =4x 06-7822 + 4x 06-7823 |
| NM21194/5 | Valve spring set, Navigator | =4x NM21194 + 4x NM21195 |

Valve Spring Cups/Cotters

| 06-7507 | Cotters (pair), all Twins |
| --- | --- |
| 06-7506 | Top Cup, valve spring, all Twins |
| 06-7545 | Bottom Cup, valve spring, all Twins up to 750cc |
| 06-1399 | Bottom Cup, valve spring, 850cc Twins |

Valves
(L= overall length from top of stem to bottom of tulip, D= diameter)

| 06-7942 | Valve, inlet, all ally head Twins pre-1960, L102,9mm, D35,7mm |
| --- | --- |
| 06-6974 | Valve, inlet, all Twins post-1960 up to 650SS, L99.9mm, D35.7mm |
| 06-7884 | Valve, inlet, Atlas, L100.2mm, D38.1mm |
| 06-4034 | Valve, inlet, Commando 750&850, L102.3mm, D37.8mm |
| 06-7570 | Valve, exhaust, all Twins pre-Commando, L99.5mm, D33.3mm, also all iron head twins, inlet |
| 06-5115 | Valve, exhaust, Commando 750&850, L101.9mm, D33.0mm |

Valve Guides, Cast Iron
We recommend to use the Inlet guides of the Commando 750 with oil seal on the earlier 500cc-750cc Twins.
These used the same guides on exhaust and inlet originally.

| 06-2726 | Oil seal, for inlet valve guide, all Twins |
| --- | --- |
| 06-2725 | Valve guide, inlet, all twins up to 750cc |
| 06-2760 | Ditto, +.002" oversize |
| 06-2767 | Ditto, +.005" oversize |
| 06-2768 | Ditto, +.010" oversize |
| 06-2769 | Ditto, +.015" oversize |
| 06-3527 | Valve guide, exhaust, all twins up to 750cc |
| 06-3528 | Ditto, +.005" oversize |
| 06-3529 | Ditto, +.010" oversize |
| 06-3530 | Ditto, +.015" oversize |
| 06-5024 | Valve guide, exhaust, all 850 Twins |
| 06-5025 | Ditto, +.002" oversize |
| 06-5026 | Ditto, +.015" oversize |
| 06-5027 | Valve guide, inlet, all 850 Twins |
| 06-5028 | Ditto, +.002" oversize |
| 06-5029 | Ditto, +.015" oversize |
| 01-1111 | Circlip for 850 Valve Guide |

Valve Guides- Bronze
Original guides were cast iron. The bronze guides are popular for racing engines
as they have better heat transfer and anti-seizure qualities.

| 06-7288 | Guide, 850 Exhaust, STD size |
| --- | --- |
| 06-7289 | Guide, 850 Exhaust, +.002" |
| 06-7290 | Guide, 850 Exhaust, +.004" |
| 06-7291 | Guide, 850 Exhaust, +.006" |
| 06-7292 | Guide, 850 Inlet, STD size |
| 06-7293 | Guide, 850 Inlet, +.002" |
| 06-7294 | Guide, 850 Inlet, +.004" |
| 06-7295 | Guide, 850 Inlet, +.006" |
| 06-7296 | Guide, 500-750 Exhaust, STD size |
| 06-7297 | Guide, 500-750 Exhaust, +.002" |
| 06-7298 | Guide, 500-750 Exhaust, +.004" |
| 06-7299 | Guide, 500-750 Exhaust, +.006" |
| 06-7300 | Guide, 500-750 Inlet, STD size |
| 06-7301 | Guide, 500-750 Inlet, +.002" |
| 06-7302 | Guide, 500-750 Inlet, +.004" |
| 06-7303 | Guide, 500-750 Inlet, +.006 |