Ch 32 – Torque Settings

These Torque settings below have been collected and agreed on by a number of respected Norton Dominator engine restorers. Some of these settings will be at odds with those recommended in maintenance manuals but we would suggest that the experience of the contributors is worth heeding. In most cases the recommendations match those found in the Likes of Garratt and Haynes manuals. The significant exceptions being those of studs that are set into Aluminium which are prone to stripping their threads. In most of these cases a slightly lower torque value has been suggested.

Description 							Torque Setting

Alternator mounting stud nuts 				15lbs.ft

Big end conrod nuts			- 500 & 600cc		15lbs.ft

- 650 & 750cc		25lbs.ft

Cam chain tensioner nuts 					15lbs.ft

Camshaft nut 							50lbs.ft

Carburettor stud nut & screws 				  8lbs.ft (more than this causes distortion)

Centre stand nuts ? 						20lbs.ft

Chainguard nut 						  8lbs.ft

Clutch-to-Gear Box mainshaft nut 				70lbs.ft

Coil mounting bracket bolts 					  8lbs/ft

Connecting rod nuts - 500/600cc 				15lbs.ft

- 650/750cc 				25lbs.ft

Crankshaft flywheel nuts 					25lbs.ft

Crankshaft oil worm nut (left-hand thread) 			25lbs.ft

Crankshaft Drive side nut 					70lbs.ft

Cylinder base nuts (3/8") 					25lbs.ft

Cylinder base nuts (5/16") 					20lbs.ft

Cylinder head nuts & bolts (3/8") 				30lbs.ft

Cylinder head nuts (5/16") 					20lbs.ft

Cylinder through sleeve & rear nut 				25lbs.ft 3/8 BSC studs into alloy (strips easily)

Control lever handlebar mounting screws 			12lbs.ft

Drum brake cam fixing nut 					30lbs.ft

Engine front plate nut 						25lbs.ft

Engine mounting bolts various 	- 1/4” 			15lbs.ft `

- 5/16” 		20lbs.ft

- 3/8” 			25lbs.ft

Engine steady centre stud nut					25lbs.ft 3/8 BSC nut to stud= 25lbs spanner on stud

Engine steady to head screw 					12lbs.ft

Exhaust Rose Nut 						50lbs.ft

Featherbed frame rear crossbar stud nuts 			30lbs.ft

Footpeg nut 							30lbs.ft

Footrest mounting bolt ? 					15lbs.ft 7/16 stud with 2 steel nuts. Spacer a must.

Footrest mounting flange nut 					  8lbs.ft

Fork damper tube anchor bolt ? 				10lbs.ft for alloy damper greater for steel damper

Fork stanchion top chrome nuts 				30lbs.ft

Fork column domed top nut 					30lbs.ft

Front brake lever pivot bolt nut 				  2lbs.ft

Front mudguard stay bolt 					10lbs.ft

Front spindle pinch bolt nut 					10lbs.ft

Front wheel spindle nut 					60lbs.ft

Fuel tank underside bolts 					12lbs.ft

Gearbox inner cover nuts 					12lbs.ft

Gearbox sprocket nut (Left-hand Thread) 			80lbs.ft

Gearbox top fixing bolt 					55lbs.ft

Handlebar mounting bolts ? 					12lbs.ft 5//16 BSF HT Allen bolts = more

Headlamp shroud bolt 					15lbs.ft

Horn nut 							  8lbs.ft

HT and coil clip bolts 						10lbs.ft

Kickstart pinch bolt ? 						25lbs.ft (or more if Allen bolt used)

Lift handle clip nut 1/4" 					  8lbs.ft

Lower yoke socket pinch screw nuts ? 			25lbs.ft

Mudguard bridge stud nut 					  8lbs.ft

Mudguard stay nut 						  4lbs.ft

Oil junction block bolt 					  8lbs.ft

Oil pressure release valve 					25lbs.ft

Oil pump stud nuts 						15lbs.ft

Oil tank mounting bolt top/bottom 				12lbs.ft

Pillion footrest pivot bolt nut 					  8lbs.ft

Pillion footrest bolt 						25lbs.ft

Prop stand clamp nut 						15lbs.ft

Rear engine plate nut 						30lbs.ft

Rear mudguard nut 						  8lbs.ft

Rear mudguard nut 1/4" 					  8lbs.ft

Rear mudguard nut 5/16" 					15lbs.ft

Rear side plate mounting nut 					25lbs.ft

Rear wheel adjuster nut 					  8lbs.ft

Rear wheel nut, dummy axle 					70lbs.ft

Rear wheel spindle 						70lbs.ft

Rectifier nut 							  3lbs.ft

Reflector nut 	(if fitted)					  3lbs.ft

Rocker feed banjo bolts 					15lbs.ft

Rocker spindle cover plate bolts 				  8lbs.ft

Rotor nut, crankshaft 						70lbs.ft

Silencer clip clamp bolt nut 					12lbs.ft

Spark plug 							15lbs.ft

Speedo cable to speedometer gearbox/head 			12lbs.ft

Steering stem nut 						30lbs.ft

Suspension unit nuts 		- 5/16” 			20lbs.ft

- 3/8” 				25lbs.ft

Swinging arm bolt nuts 					50lbs.ft

Tail lens pillar screw 						  2lbs.ft

Zener diode nut 						  2lbs.ft

The above setting numbers are only recommended. Studs, nuts and bolts made from non-standard metals are bound to have differing values.

For mild steel fasteners (28 U.T.S) the general rules for maximum torque values are……

¼” BSC 6lbs.ft

5/16”BSC 11lbs.ft

3/8”BSC 20lbs.ft

For high tensile strength (Type “R”) steel fasteners (45 U.T.S) the general rules for maximum torque values are……

¼” BSC 9lbs.ft

5/16”BSC 19lbs.ft

3/8”BSC 32lbs.ft

Torque settings for larger thread sizes will depend upon the application.