Ch 25 – Torque Settings

These Torque settings below have been collected and agreed on by a number of respected Norton Dominator engine restorers. Some of these settings will be at odds with those recommended in maintenance manuals but we would suggest that the experience of the contributors is worth heeding. In most cases the recommendations match those found in the Likes of Garratt and Haynes manuals. The significant exceptions being those of studs that are set into Aluminium which are prone to stripping their threads. In most of these cases a slightly lower torque value has been suggested.

Description 							Torque Setting

Alternator mounting stud nuts 				15lbs.ft

Big end conrod nuts			- 500 & 600cc		15lbs.ft

- 650 & 750cc		25lbs.ft

Cam chain tensioner nuts 					15lbs.ft

Camshaft nut 							50lbs.ft

Carburettor stud nut & screws 				  8lbs.ft (more than this causes distortion)

Centre stand nuts ? 						20lbs.ft

Chainguard nut 						  8lbs.ft

Clutch-to-Gear Box mainshaft nut 				70lbs.ft

Coil mounting bracket bolts 					  8lbs/ft

Connecting rod nuts - 500/600cc 				15lbs.ft

- 650/750cc 				25lbs.ft

Crankshaft flywheel nuts 					25lbs.ft

Crankshaft oil worm nut (left-hand thread) 			25lbs.ft

Crankshaft Drive side nut 					70lbs.ft

Cylinder base nuts (3/8") 					25lbs.ft

Cylinder base nuts (5/16") 					20lbs.ft

Cylinder head nuts & bolts (3/8") 				30lbs.ft

Cylinder head nuts (5/16") 					20lbs.ft

Cylinder through sleeve & rear nut 				25lbs.ft 3/8 BSC studs into alloy (strips easily)

Control lever handlebar mounting screws 			12lbs.ft

Drum brake cam fixing nut 					30lbs.ft

Engine front plate nut 						25lbs.ft

Engine mounting bolts various 	- 1/4” 			15lbs.ft `

- 5/16” 		20lbs.ft

- 3/8” 			25lbs.ft

Engine steady centre stud nut					25lbs.ft 3/8 BSC nut to stud= 25lbs spanner on stud

Engine steady to head screw 					12lbs.ft

Exhaust Rose Nut 						50lbs.ft

Featherbed frame rear crossbar stud nuts 			30lbs.ft

Footpeg nut 							30lbs.ft

Footrest mounting bolt ? 					15lbs.ft 7/16 stud with 2 steel nuts. Spacer a must.

Footrest mounting flange nut 					  8lbs.ft

Fork damper tube anchor bolt ? 				10lbs.ft for alloy damper greater for steel damper

Fork stanchion top chrome nuts 				30lbs.ft

Fork column domed top nut 					30lbs.ft

Front brake lever pivot bolt nut 				  2lbs.ft

Front mudguard stay bolt 					10lbs.ft

Front spindle pinch bolt nut 					10lbs.ft

Front wheel spindle nut 					60lbs.ft

Fuel tank underside bolts 					12lbs.ft

Gearbox inner cover nuts 					12lbs.ft

Gearbox sprocket nut (Left-hand Thread) 			80lbs.ft