| Dominator Engine – Ignition Timing Settings |  |  |  |  |
| --- | --- | --- | --- | --- |
| Norton Engine |  |  | Timing *BTDC | Timing inch / mm |
| Model | Year | CC | Full Advance | Full Advance |
| 7 & 88 (i) | 1949 – 54 | 497 | 31 | 1/4 or 6.35 |
| 88 & 88SS | 1955 - 66 | 497 | 30 | 1/4 or 6.35 |
| 77, 99 & 99SS (ii) | 1956 - 62 | 596 | 32 | 9/32 or 7.14 |
| 650, 650SS & Mercury | 1961 - 70 | 646 | 32 | 11/32 or 8.69 |
| Atlas & Hybrids | 1962 - 68 | 745 | 32 | 11/32 or 8.69 |
| Atlas with 8.9:1 Pistons | 1962 - 68 | 745 | 28 | 7/32 or 5.55 |

*    BTDC  (Before Top Dead Centre)  – Piston at highest point.

These Ignition Timings were sourced from a number of Norton Maintenance Manuals.

The apparent discrepancies are most likely due to variations in piston crown height.

The introduction of Lead-free fuel along with the addition of Ethanol to most petrol stocks makes the above settings a good starting point but not necessarily the optimum timing point any longer. Retarding the ignition by up to 2 degrees may be required to prevent issues with detonation. Fuel additives are also available to help restore lack of performance caused by current forecourt fuel.

Notes: (i) Engines with Iron Cylinder Heads. 30* BTDC is recommended.

(ii) Haycroft recommends 5/16” or 7.80mm.